Autonomous Drone Navigation and Obstacle Avoidance using Deep Reinforcement Learning (DRL)

Graduation Project 1 Report

Prepared by:

October 2025

MOUHAMAD ALIM AL-AMINE, 411034763

ABDUL SHAKOR MOHAMMAD AMIR, 422038873

Supervisor:

Prof. AHMAD ALKHODRE

Table of Contents

Acronyms and Abbreviations	2

Abstract	3

Chapter 1: Introduction and Project Foundation	4

1.1. Introduction and Motivation	4

1.2. Application and Usage of Autonomous Drones	4

1.3. Project Scope and Objectives	4

Chapter 2: Literature Review and Platform Analysis	5

2.1. Deep Reinforcement Learning (DRL) Overview	5

2.1.1. Policy-Gradient Methods	5

2.2. Comparative Simulator Analysis	6

2.3. Classical Control Baseline	6

Chapter 3: Methodology and Implementation	7

3.1. System Architecture and DRL Pipeline	7

3.2. Webots Environment and Plant Calibration	8

3.3. Deep Reinforcement Learning Design	8

3.3.1. Action Space (A)	8

3.3.2. Observation Space (S)	8

3.3.3. Reward Function (R)	8

Chapter 4: Results and Evaluation (Planned)	9

Chapter 5: Conclusion and Future Work	9

5.1. Conclusion	9

5.2. Future Work: FPGA Policy Deployment	10

References	11

# Acronyms and Abbreviations

| Acronym | Full Name | Description |
| --- | --- | --- |
| A | Action Space | The set of all possible actions the DRL agent can take. |
| CV | Computer Vision | A field of AI that enables machines to interpret and understand visual information. |
| DQL | Deep Q-Learning | A value-based DRL algorithm that uses a neural network to approximate the Q-function. |
| DRL | Deep Reinforcement Learning | A subfield of ML combining RL algorithms with deep neural networks. |
| FPGA | Field-Programmable Gate Array | An integrated circuit designed to be configured by a customer or designer after manufacturing; used for hardware-accelerated policy inference. |
| GPS | Global Positioning System | A satellite-based navigation system used to determine location. |
| IMU | Inertial Measurement Unit | An electronic device that measures and reports a body's specific force, angular rate, and orientation. |
| S | Observation/State Space | The complete set of information received by the agent about the environment. |
| R | Reward Function | The scalar signal used to evaluate the desirability of the agent's actions. |
| MDP | Markov Decision Process | The mathematical framework for modeling decision making in situations where outcomes are partly random and partly under the control of a decision maker. |
| PPO | Proximal Policy Optimization | A state-of-the-art policy-gradient DRL algorithm, known for stability and sample efficiency. |
| SAR | Search and Rescue | Operations performed to save people in distress. |
| SLAM | Simultaneous Localization and Mapping | Computational problem of constructing or updating a map of an unknown environment while simultaneously keeping track of an agent's location within it. |
| UAV | Unmanned Aerial Vehicle | An aircraft without a human pilot aboard, commonly referred to as a drone. |
| VIO | Visual-Inertial Odometry | A technology that estimates the pose of an agent using a combination of visual (camera) and inertial (IMU) sensor data. |
| Webots | Webots | The selected open-source 3D robot simulator. |
| ω0 | Base Hover Velocity | The calibrated motor velocity required to maintain stable hover against gravity (T = m ⋅g). |

# Abstract

This project focuses on developing an intelligent, autonomous quadrotor system capable of navigation and robust obstacle avoidance in complex environments. By utilizing Deep Reinforcement Learning (DRL), specifically the Proximal Policy Optimization (PPO) algorithm, an agent is trained in a high-fidelity simulation environment (Webots) to learn an end-to-end control policy. This policy directly maps sensor inputs (IMU, GPS, Distance Sensors) to motor commands. The core contribution is the design and implementation of the DRL S, A, R  (State, Action, Reward) framework optimized for continuous control, including the creation of a custom Webots-Gymnasium environment wrapper to bridge the simulation and DRL algorithm. The project's methodology centers on establishing a stable simulation-to-learning pipeline, with future work focusing on the feasibility of hardware-accelerated policy deployment via Field-Programmable Gate Array (FPGA). This report details the system architecture, DRL design choices, and initial environment calibration efforts.

# Chapter 1: Introduction and Project Foundation

## 1.1. Introduction and Motivation

The proliferation of Unmanned Aerial Vehicles (UAVs), particularly quadrotors, has expanded their utility across civilian and industrial sectors. To transition from manual control to reliable autonomy, UAVs require robust decision-making systems that can handle dynamic, unstructured environments. Traditional control methods (e.g., PID-based path planning) struggle with high-dimensional state spaces and real-time adaptation.

This project addresses this limitation by proposing a solution based on Deep Reinforcement Learning (DRL). DRL allows an agent to learn optimal control policies directly through trial-and-error interaction within a simulated environment, bypassing the need for explicit environment modeling [1]. The goal is to develop an adaptive navigation policy that ensures successful trajectory completion while minimizing control effort and avoiding obstacles.

## 1.2. Application and Usage of Autonomous Drones

Autonomous quadrotors have transformative applications requiring precise and adaptive flight control [2]:

- Package Delivery and Logistics: Utilizing drones for last-mile delivery requires complex navigation in urban or uneven terrains, demanding high obstacle avoidance reliability.

- Search and Rescue (SAR): Drones can autonomously patrol disaster areas, requiring the ability to navigate through cluttered, unknown spaces to locate targets.

- Infrastructure Inspection: Autonomous flight along complex structures (bridges, wind turbines) requires precise attitude control and continuous obstacle detection.

- Precision Agriculture (AgriTech): Drones equipped with Computer Vision (CV) systems can perform high-resolution aerial imaging of crops. By applying image processing and machine learning techniques, these systems can autonomously detect and map early signs of plant disease, pest infestations, or nutritional deficiencies across large farms, enabling targeted intervention and optimizing yields.

## 1.3. Project Scope and Objectives

The project is divided into two phases:

1.	Phase 1 (Core Deliverable): Design and implement the DRL framework within the Webots simulation environment, train the PPO agent to successfully navigate a constrained 3D course, and establish performance benchmarks.

2.	Phase 2 (Extended Goal): Investigate the feasibility of deploying the trained neural network policy onto an FPGA for low-latency, real-time inference, which serves as a hardware-in-the-loop validation of the system's practical applicability [3].

# Chapter 2: Literature Review and Platform Analysis

## 2.1. Deep Reinforcement Learning (DRL) Overview

DRL is rooted in the Markov Decision Process (MDP) framework, defined by a set of states (S), actions (A), a transition function (T), and a reward function (R) [1].

### 2.1.1. Policy-Gradient Methods

For continuous control tasks like quadrotor flight, Policy-Based algorithms are preferred over Value-Based methods (like DQN [4]). Proximal Policy Optimization (PPO) is selected due to its stability, sample efficiency, and robust handling of continuous action spaces, making it an industry-standard for robotic control [5].

## 2.2. Comparative Simulator Analysis

The choice of simulation environment is critical, as it directly impacts training speed and the Sim-to-Real gap [6].

| Simulator | Key Features | Rationale for Selection (Webots) |
| --- | --- | --- |
| AirSim | High-fidelity photorealism, good for visual SLAM/VIO. | Focuses heavily on graphical rendering, often leading to slower training loops for non-visual tasks [7]. |
| Gazebo | Industry standard, strong community, complex physics modeling. | Known for complexity in initial setup and overhead, sometimes less ideal for quick DRL iteration cycles. |
| Webots | Lightweight, native Python API, robust sensor modeling, and explicit support for robotics/embedded systems [8]. | Selected platform. It provides the optimal balance of physics fidelity and computational efficiency required for millions of DRL steps. Crucially, its design allows for straightforward abstraction of the physics model, which is necessary for the subsequent FPGA objective. |

## 2.3. Classical Control Baseline

Initial control efforts utilized a simplified P-D controller structure for attitude stabilization. This phase revealed the high sensitivity of the simulated quadrotor to thrust variations. This exercise proved the necessity of establishing a stable HOVER_VELOCITY (ω0) before DRL training, confirming the complexity of the physical dynamics the DRL agent must master.

# Chapter 3: Methodology and Implementation

The DRL methodology is built around defining the three core elements of the MDP: S, A, and R.

## 3.1. System Architecture and DRL Pipeline

The training system is structured as a pipeline facilitating synchronous communication between the machine learning and physics environments.

●	DRL Agent (Python/PyTorch): Implements the PPO algorithm.

●	Gymnasium Environment Wrapper: A custom Python class that encapsulates the Webots control loop, translating DRL actions to physical motor commands and sensor readings to DRL states.

Figure 1: A diagram showing the DRL Pipeline

## 3.2. Webots Environment and Plant Calibration

The physical model must be precisely calibrated to ensure the agent learns a transferable policy.

●	Calibration Status: The primary challenge identified is the precise HOVER_VELOCITY (ω0) The current focus is on manual tuning to find the value where the total thrust exactly equals the force of gravity (T = m ⋅g), resulting in zero net vertical acceleration (vz≈0) .

●	Implementation Strategy: This calibrated ω0 will be set as the base thrust within the Gymnasium reset() and step() functions. The DRL agent's output will only modify this base value, significantly reducing the complexity of the learning problem.

## 3.3. Deep Reinforcement Learning Design

### 3.3.1. Action Space (A)

The action space is designed to give the agent control over the quadrotor's attitude and collective thrust, using rates of change for stability.

●	Type: Boxlow=-1.0, high=1.0, Dimension 4.

●	Vector Definition At=ϕ,θ,az,ψ:

1.	ϕ (Roll Rate Command): Desired roll rate change (lateral movement).

2.	θ (Pitch Rate Command): Desired pitch rate change (forward/backward movement).

3.	az (Vertical Acceleration Adjustment): Normalized thrust adjustment relative to ω0.

4.	ψ (Yaw Rate Command): Desired angular velocity around the Z-axis.

### 3.3.2. Observation Space (S)

The state vector provides all necessary information for the agent to compute its next optimal action, derived from the IMU and GPS sensors.

●	Type: Boxlow=-∞, high=∞, Dimension 14.

●	Vector Definition St:

1.	Goal Vector (3): Δx, Δy, Δz (Relative position to the target).

2.	Attitude (3): Roll (ϕ), Pitch (θ), Yaw (ψ).

3.	Linear Velocity (3): Vx, Vy, Vz (from GPS).

4.	Angular Velocity (3): ωx,ωy,ωz (from IMU).

5.	Distance Sensors (2): Front distance dfront, and absolute distance to goal dtarget.

### 3.3.3. Reward Function (R)

The composite reward function guides the agent towards the goal while penalizing inefficient and unstable flight.

Rtotal=RGoal+RSurvival+REffort

| Term | Description | Objective |
| --- | --- | --- |
| RGa | Proportional to the reduction in distance to the target at each step. A large positive bonus (+1000) on successful goal arrival. | Primary drive for purposeful navigation. |
| RSria | Large negative terminal reward (-5000) upon collision with an obstacle, the ground, or exceeding designated altitude limits. | Enforce collision avoidance and safety boundaries. |
| REfr | Small negative reward proportional to the squared magnitude of the action vector (¦At¦2). | Encourage efficient, minimal-effort control inputs. |

Chapter 3: Methodology and Implementation (Continued)

3.4. Baseline Control System: PID Implementation and Stabilization

A fundamental requirement for training a high-level DRL navigation agent is a stable, low-level control platform. The primary objective of this phase was to implement a robust Proportional (P) controller that could achieve a stable, hands-off hover at a target altitude of 1.0 meter. This low-level controller forms the stable "inner loop" that the DRL agent will later command.

3.4.1. Analytical Thrust Calibration (HOVER_VELOCITY, $\omega_0$)

Stability was achieved by analytically determining the Base Hover Velocity ($\omega_0$), the constant motor speed required to produce a thrust ($T$) that exactly counteracts the drone’s weight ($m \cdot g$). This prevented the catastrophic ascent observed in initial empirical trials. The hover velocity is calculated based on the drone’s mass ($m_{drone}$) and the motor thrust constant ($K$):

$$T_{total} = 4 \cdot K \cdot \omega_0^2 = m_{drone} \cdot g$$

This physics-based approach provides the DRL agent with a "near-zero" baseline vertical thrust command, allowing the agent to focus its learning capacity entirely on attitude and horizontal navigation adjustments.

3.4.2. Motor Mixing and Control Architecture

The PID-like controller utilizes a P-term for altitude ($K_{P,alt}$) and a separate P-term for attitude correction ($K_{P,att}$). The final velocity command for each of the four motors ($\omega_{i}$) is a mix of the collective thrust (for vertical movement) and the differential thrusts (for roll, pitch, and yaw):

$$\omega_{i} = \omega_{base} + \omega_{alt} \pm \omega_{roll} \pm \omega_{pitch} \pm \omega_{yaw}$$

Crucially, the sign convention for the yaw control component was meticulously debugged and inverted to match the quadrotor's physical torque direction in the Webots environment. This correction resolved the violent, uncontrollable instability that plagued the initial implementation.

$$\omega_{\text{collective}} = \omega_0 + (K_{P,alt} \cdot \text{Altitude Error}) + (K_{D,alt} \cdot \text{Vertical Velocity})$$

The system utilizes the GPS sensor's vertical velocity reading as a strong derivative (D) term for altitude, providing essential damping to prevent overshoot and oscillation.

Chapter 4: Initial Results and Baseline Evaluation

4.1. Baseline Performance: Stable Hover Metrics

The successful implementation and stabilization of the baseline controller represent the first major milestone of the project. The metrics below establish the minimum performance standard that the high-level DRL agent must meet or exceed.

| Metric | Target Value | Achieved (PID) | Notes |
| --- | --- | --- | --- |
| Hover Altitude ($Z$) | 1.0 m | $1.00 \pm \text{[Insert Measured Fluctuation]}$ m | Final, stable altitude. |
| Max Altitude Overshoot | Minimal | $\text{[Insert Measured Max Overshoot]}$ m | Peak height reached during takeoff/stabilization. |
| Max Attitude Deviation | Minimal | $\text{[Insert Measured Max Roll/Pitch Angle]}$ rad | Maximum tilt observed during stable hover. |
| Stabilization Time ($t_{90\%}$) | $< 5$ s | $\text{[Insert Measured Time]}$ s | Time to settle within 10% of target altitude. |

Visualization of Stable Hover

The figure below illustrates the drone's position along the Z-axis over the first 10 seconds of flight, demonstrating a smooth takeoff, minimal overshoot, and effective dampening leading to a stable hover. This confirms the robustness of the low-level control system.

Chapter 5: Extended Goals and Future Work (Revised)

5.1. Transition to Deep Reinforcement Learning (DRL)

With the stable baseline achieved, the immediate next phase is the implementation of the Webots-Gymnasium Wrapper. This wrapper will treat the PID-controlled drone as a stable physical plant, allowing the DRL agent to interact with the system via high-level commands (e.g., target roll/pitch angles and velocity adjustments).

5.2. Consideration for Hardware Implementation (FPGA)

Upon successful training and validation of the DRL policy in simulation, an extended goal remains the investigation of hardware-accelerated inference using an FPGA. This step would involve quantizing the trained neural network model [1] [3] to compile the quantized model for efficient inference on a target FPGA (e.g., a Altera DE1 SoC platform). This step validates the DRL solution for real-time operation in embedded systems, offering significant advantages in power efficiency and latency over traditional CPU-based inference [3].

# Chapter 5: Conclusion and Future Work

## 5.1. Conclusion

This report details the foundational methodology for developing a DRL-based autonomous quadrotor system. The initial phase successfully established the Webots-Gymnasium environment and defined the S, A, R  framework using the stable PPO algorithm. The project is strategically positioned to begin the core training phase immediately upon final system calibration (i.e., identifying HOVER_VELOCITY).

## 5.2. Future Work: FPGA Policy Deployment

The project's extended goal involves addressing the constraint of real-world low-latency control by investigating hardware-accelerated policy execution. This involves:

1.	Quantization: Converting the trained DRL neural network (NN) from floating-point (FP32) weights to fixed-point (INT8) representation.

2.	Deployment: Using tools like Xilinx Vitis AI [1] [3] to compile the quantized model for efficient inference on a target FPGA (e.g., a Altera DE1 SoC platform).

This step validates the DRL solution for real-time operation in embedded systems, offering significant advantages in power efficiency and latency over traditional CPU-based inference [3].

# References

| [1] | R. S. Sutton and A. G. Barto, Reinforcement Learning: An Introduction, 2nd ed., Cambridge: MA: The MIT Press, 2018. |
| --- | --- |
| [2] | J. Saunders, S. Saeedi, and W. Li, "Autonomous aerial robotics for package delivery: A technical review," Field Robotics, 2023. |
| [3] | B. Kövari and E. Ebeid, "MPDrone: FPGA-based Platform for Intelligent Real-time Autonomous Drone Operations," 2021 IEEE International Symposium on Safety, Security, and Rescue Robotics (SSRR), pp. 71-76, 2021. |
| [4] | M. Hessel et al, "Rainbow: Combining Improvements in Deep Reinforcement Learning," arXiv, 2017. |
| [5] | AlMahamid and K. Grolinger, "Reinforcement Learning Algorithms: An Overview and Classification," arXiv, 2022. |
| [6] | M. Nikolaiev and M. Novotarskyi, "COMPARATIVE REVIEW OF DRONE SIMULATORS," Information, Computing and Intelligent Systems,, vol. 4, pp. 79-99, 2024. |
| [7] | J. Saunders, S. Saeedi, and W. Li, "Parallel Reinforcement Learning Simulation for Visual Quadrotor Navigation," Arxiv, September 2022. |
| [8] | Michel, O., "Webots: Professional Mobile Robot Simulation," Journal of Advanced Robotics Systems, vol. 1, pp. 39-42, 2004. |